1️⃣ Power Supply Table

| Component | Power Source | Voltage | Notes / Safety |
| --- | --- | --- | --- |
| Arduino UNO R3 | 12V DC Adapter | 12V | Connect to VIN or DC jack. Regulated to 5V for sensors. |
| SIM800L GSM Module | 3.7V Li-ion battery | 3.7V–4.2V | Do NOT power from Arduino 5V. Common GND with Arduino required. |

2️⃣ Sensor Pin Table

| Sensor | Arduino Pin | Type | Power | Notes |
| --- | --- | --- | --- | --- |
| Flame Sensor | A0 | Analog | 5V | Detects fire. Adjust threshold in code (500 used). |
| Soil Moisture Sensor | A1 | Analog | 5V | Detects landslide possibility by dry soil. |
| Tilt Sensor (Earthquake) | D6 | Digital | 5V | Detects vibration / tilting of surface. HIGH = detected. |
| Ultrasonic Sensor (HC-SR04) | TRIG → D4, ECHO → D5 | Digital | 5V | Measures distance to detect landslide risk (distance < 10cm). |

3️⃣ Output Devices Table

| Device | Arduino Pin | Power | Notes |
| --- | --- | --- | --- |
| Buzzer | D8 | 5V | Beeps when any disaster is detected. Active type. |

4️⃣ SIM800L GSM Module Table

| SIM800L Pin | Arduino Pin / Connection | Notes |
| --- | --- | --- |
| TX | D10 → Arduino RX | Send data from SIM800L to Arduino |
| RX | D11 → Arduino TX | Receive data from Arduino to SIM800L |
| PWR KEY | D9 | Control power on/off via Arduino |
| VCC | 3.7V battery + | Use proper battery. 2A peak current required. |
| GND | Common GND | Connect both Arduino GND and Battery – |
| Antenna | Attached | Required for network connectivity |

5️⃣ Disaster Trigger Logic Table

| Disaster Type | Trigger Condition | Output Action |
| --- | --- | --- |
| Fire | Flame sensor > 500 | Buzzer ON + SMS + Call |
| Landslide | Soil < 300 AND Distance < 10cm | Buzzer ON + SMS + Call |
| Earthquake | Tilt sensor HIGH | Buzzer ON + SMS + Call |

6️⃣ SMS + Call Numbers Table

| Type | Phone Number |
| --- | --- |
| SMS + Call | +639935380795 |
| SMS + Call | +97798XXXXXXXX |

7️⃣ Notes for Safe Operation

| Item | Recommendation |
| --- | --- |
| SIM800L Power | Must use 3.7–4.2V Li-ion battery. Do not use Arduino 5V. |
| GND | All modules must share common GND (Arduino + Battery + Sensors + SIM800L). |
| Antenna | Attach to SIM800L for stable SMS and call. |
| Testing | Test each sensor individually before full system test. |
| Code Thresholds | Flame = 500, Soil = 300, Distance = 10cm (adjust after testing). |

Code

// ================= PIN DEFINITIONS =================

#define FLAME A0          // Flame sensor analog input

#define SOIL A1           // Soil moisture analog input

#define TRIG 4            // Ultrasonic trigger

#define ECHO 5            // Ultrasonic echo

#define TILT 6            // Tilt sensor digital input

#define BUZZER 8          // Buzzer output

#define SIM_TX 10         // SIM800L TX

#define SIM_RX 11         // SIM800L RX

#define SIM_PWR 9         // SIM800L PWR KEY

#include <SoftwareSerial.h>

SoftwareSerial sim800(SIM_TX, SIM_RX);

// ================= SETUP =================

void setup() {

pinMode(FLAME, INPUT);

pinMode(SOIL, INPUT);

pinMode(TILT, INPUT);

pinMode(TRIG, OUTPUT);

pinMode(ECHO, INPUT);

pinMode(BUZZER, OUTPUT);

pinMode(SIM_PWR, OUTPUT);

Serial.begin(9600);

sim800.begin(9600);

// Turn on SIM800L

digitalWrite(SIM_PWR, HIGH);

delay(3000);

Serial.println("SYSTEM READY");

}

// ================= MAIN LOOP =================

void loop() {

int flameVal = analogRead(FLAME);

int soilVal = analogRead(SOIL);

int tiltVal = digitalRead(TILT);

long distance = measureDistance();

bool disasterDetected = false;

String alertMessage = "";

// Check Flame

if (flameVal > 500) {

disasterDetected = true;

alertMessage = "🔥 FIRE ALERT: Fire detected!";

}

// Check Landslide

else if (soilVal < 300 && distance < 10) {

disasterDetected = true;

alertMessage = "🌧 LANDSLIDE WARNING: Soil loose!";

}

// Check Earthquake

else if (tiltVal == HIGH) {

disasterDetected = true;

alertMessage = "🌍 EARTHQUAKE MOTION DETECTED!";

}

if (disasterDetected) {

// Buzzer ON

digitalWrite(BUZZER, HIGH);

Serial.println(alertMessage);

// Send SMS + Call to two numbers

sendSMS(alertMessage);

makeCall();

// Keep buzzer ON for 12 seconds

delay(12000);

digitalWrite(BUZZER, LOW);

} else {

digitalWrite(BUZZER, LOW);

}

delay(1000); // small delay between readings

}

// ================= ULTRASONIC DISTANCE FUNCTION =================

long measureDistance() {

digitalWrite(TRIG, LOW);

delayMicroseconds(2);

digitalWrite(TRIG, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG, LOW);

long duration = pulseIn(ECHO, HIGH);

return duration * 0.034 / 2; // Distance in cm

}

// ================= SEND SMS TO TWO NUMBERS =================

void sendSMS(String msg) {

sendText("+639935380795", msg); // Replace with your first number

sendText("+97798XXXXXXXX", msg); // Replace with your second number

}

void sendText(String number, String message) {

sim800.println("AT"); // Test module

delay(500);

sim800.println("AT+CMGF=1"); // Set SMS text mode

delay(800);

sim800.println("AT+CMGS=\"" + number + "\""); // Recipient

delay(800);

sim800.println(message); // Message content

delay(500);

sim800.write(26); // CTRL+Z to send

delay(4000);

Serial.println("📩 SMS sent to " + number);

}

// ================= CALL TWO NUMBERS =================

void makeCall() {

callNumber("+639935380795"); // First number

callNumber("+97798XXXXXXXX"); // Second number

}

void callNumber(String number) {

sim800.println("ATD" + number + ";"); // Dial number

Serial.println("📞 Calling " + number);

delay(15000); // Ring 15 sec

sim800.println("ATH"); // Hang up

delay(1500);

}