OBJECT:

To design and develop a working prototype capable of detecting natural disaster indicators such as landslide movement, soil moisture, fire outbreak, and distance change using sensors, and alerting users through alarm and GSM messages.

MATERIALS REQUIRED

- Arduino Uno / Mega

- Soil Moisture Sensor

- Ultrasonic Sensor (HC-SR04)

- Tilt Sensor

- Flame Sensor

- Buzzer

- GSM Module (SIM800L/SIM900)

- Jumper wires and breadboard

- Power supply / USB cable

- SIM card with active network

- Smartphone for receiving alerts

THEORY

Natural disasters such as landslides, fires, and floods occur suddenly but show early warning signs.
Sensors help detect these changes in the environment:

- Soil Moisture Sensor
Measures water content in the soil. Sudden saturation increases chances of landslides.

- Tilt Sensor
Detects vibration or angular movement. Continuous shifts indicate ground instability.

- Ultrasonic Sensor
Measures distance using sound waves. Changing distance may indicate soil displacement, rock fall, or mud movement.

- Flame Sensor
Detects infrared radiation produced by fire and alerts early before fire spreads.

- GSM Module
Allows the system to send SMS alerts to the assigned phone number when any abnormal condition is detected.

The Arduino processes input data from sensors, compares them with threshold values, and triggers alarms and SMS alerts.
This early warning helps minimize disaster risks and gives time for evacuation.

BUILDING / PROCEDURE

- Connect all sensors to the Arduino following standard wiring:

  - Soil Moisture → Analog Input A0

  - Flame Sensor → Digital Pin 2

  - Tilt Sensor → Digital Pin 3

  - Ultrasonic Sensor → Trig Pin 4 and Echo Pin 5

  - Buzzer → Pin 6

  - GSM Module → Software Serial Pins 8 (RX) & 9 (TX)

- Power the Arduino through USB or 9V battery.
Connect GSM module to external 3.7–4.2V battery if required.

- Upload the Arduino program with threshold values assigned for:

  - Soil moisture saturation

  - Flame detection

  - Tilt movement

  - Distance variations

- Insert an active SIM card into the GSM module and wait for signal.

- Test each sensor:

  - Wet soil near the moisture sensor

  - Tilt the tilt switch

  - Bring flame near flame sensor

  - Obstruct ultrasonic sensor

- Observe:

  - Buzzer turns on when danger value crosses threshold

  - SMS alert is received on mobile

OBSERVATION

- Moist soil causes the system to activate alert.

- Tilt of ground triggers vibration alarm.

- Fire is detected immediately.

- Sudden distance change indicates sliding surface or falling object.

- GSM sends real-time warning messages.

CONCLUSION

An Integrated Disaster Management System was successfully designed and tested.
The project proved that combining multiple low-cost sensors with Arduino can detect early warning signs of natural disasters.
Such systems are suitable for landslide-prone areas, rural monitoring, and school safety initiatives.
Early alerts help reduce loss of life and property, showing that modern technology can play a crucial role in disaster risk reduction.